# Assignment 4:
Interrupts and Timers

Cal Poly CPE 329-01

Spring 2017

Brendan Baronia

Melinda Ong

Professor Gerfen, Jeffrey B.

April 21, 2017

Clock Calculation Arithmetic:

24 KHz 25% duty Clock:

DC0=24 MHz

CLK = 25 KHz

Cycles=DC0CLK=24 MHz25 KHz=960

HIGH = 960 *0.25=240 cycles

Thi=240 cycles / 24 MHz=10 μs

LOW = 960 * 0.75 = 720 cycles

Tlo=720 cycles / 24 MHz = 30 μs

T=960 cycles / 24 MHz =40 μs

To implement the 25% duty cycle, we can leave LED on for 1 interrupt, off for 3 interrupts

CCR0=240

24 KHz 50% duty Square Wave:

DC0=24 MHz

CLK = 25 KHz

Cycles=DC0CLK=24 MHz25 KHz=960

HIGH = 960 *0.5=480 cycles

Thi=240 cycles / 24 MHz=20 μs

LOW = 960 * 0.5 = 480 cycles

Tlo=720 cycles / 24 MHz = 20 μs

T=960 cycles / 24 MHz =40 μs

CCR0 =480

20 second 50% duty Square Wave:

T=20 s

DC0=1.5 MHz

CLK =120 s= 50 mhz

Cycles=DC0CLK=1.5 MHz50 mhz=30000000

HIGH = 30000000 *0.5=15000000 cycles

Thi=15000000 cycles / 1.5 MHz=10 s

LOW = 30000000 * 0.5 = 15000000 cycles

Tlo=15000000 cycles / 1.5 MHz = 10 s

By counting to 500 interrupts, we can implement a 20 second period without the counter rolling over.

HIGH =250 interrupts

LOW = 250 interrupts

30000000 cycles500 interrupts=60000 cycles per interrupt

CCR0 = 60000

2 Bit Timer:

DC0=1.5 MHz

CLK1 = 500 Hz

CLK0 = 1000Hz

Cycle1 = 1.5 Mhz500 Hz=3000

Cycle0 = 1.5 Mhz1000Hz=1500

CCR1 = 1500

CCR0 = 750

Oscilloscope Screenshots:

Step 2.) 25 KHz Clock with 25% Duty Cycle

Figure 1: 25KHz clock (step 2) with a 25% duty cycle using DCO = 24MHz

Step 4.) ISR Processing Measurement

Figure 2: ISR execution and MCLK (step 4)

Figure 3: [Zoomed] ISR execution and MCLK (step 4)

In Figure 2, a period of 2.283 microseconds is observed to execute an empty interrupt. In Figure 3, a period of 43.000 nanoseconds is observed to execute a single MCLK cycle.

Number of Cycles=2.283 x 10-6 seconds43 x 10-9 seconds=53.09

Therefore, it takes approximately 53 MCLK cycles for ISR execution to measure a bit.

Step 5.) Shortest Pulse and Failing ISR

Figure 4: Shortest pulse (step 5)

Figure 5: [Zoomed] Shortest pulse (step 5)

In Figure 4, a period of 3.8020 microseconds and frequency of 263.02 kHz is observed for the shortest pulse.

Step 6.) Clock with 50% Duty Cycle and 20-Second Period

Youtube video of 20-second clock: https://youtu.be/20bfWFgdquA

Step 8.) 2-Bit Counter

Figure 6: 2-bit counter (step 8)

Figure 7: [Zoomed] 2-bit counter (step 8)

In Figure 7, a delay of 46 microseconds is observed between Timer_A0 CCR0 and Timer_A0 CCR1.

Code:

i.) main.c with ISR for 25 KHz clock system

//******************************************************************************
//  MSP432P401 Demo - Timer0_A3, Toggle P1.0, CCR0 Cont Mode ISR, DCO SMCLK
//
//  Description: Toggle P1.0 using software and TA_0 ISR. Timer0_A is
//  configured for continuous mode, thus the timer overflows when TAR counts
//  to CCR0. In this example, CCR0 is loaded with 50000.
//  ACLK = n/a, MCLK = SMCLK = TACLK = default DCO = ~3MHz
//
//           MSP432P401x
//         ---------------
//     /|\|               |
//      | |               |
//      --|RST            |
//        |               |
//        |           P1.0|-->LED
//
//   William Goh
//   Texas Instruments Inc.
//   June 2016 (updated) | November 2013 (created)
//   Built with CCSv6.1, IAR, Keil, GCC
//******************************************************************************
#include "msp.h"

int main(void) {
    WDT_A->CTL = WDT_A_CTL_PW |             // Stop WDT
            WDT_A_CTL_HOLD;

    // Configure P1
    P1->SEL1 &= ~BIT0;
    P1->SEL0 &= ~BIT0;
	P1->DIR |= BIT0;
	
	CS->KEY = CS_KEY_VAL; // unlock CS registers
    CS->CTL0 = 0; // clear register CTL0
	CS->CTL0=CS_CTL0_DCORSEL_4; //24 MHz
	CS->CTL1 = CS_CTL1_SELA_2 | CS_CTL1_SELS_3 | CS_CTL1_SELM_3; // select clock sources
    CS->KEY = 0; // lock the CS registers
	
	
	P1->OUT |= BIT0;
	
    TIMER_A0->CCTL[0] = TIMER_A_CCTLN_CCIE; // TACCR0 interrupt enabled
	
    TIMER_A0->CCR[0] = 240; //Set CCR0 to 240 for on cycle
	
    TIMER_A0->CTL = TIMER_A_CTL_SSEL__SMCLK | // SMCLK, continuous mode
            TIMER_A_CTL_MC__CONTINUOUS;

    SCB->SCR |= SCB_SCR_SLEEPONEXIT_Msk;    // Enable sleep on exit from ISR

    // Enable global interrupt
    __enable_irq();

    NVIC->ISER[0] = 1 << ((TA0_0_IRQn) & 31);

    while (1)
    {
        __sleep();

        __no_operation();                   // For debugger
    }
}

// Timer A0 interrupt service routine

void TA0_0_IRQHandler(void) {
    TIMER_A0->CCTL[0] &= ~TIMER_A_CCTLN_CCIFG;
	
	//Check whether LED is on or off
    if (P1->OUT & 1) { //LED ON
		TIMER_A0->CCR[0] += 720; //off timer
		P1->OUT &= ~BIT0; //turn led off
	}
	else { // LED OFF
		TIMER_A0->CCR[0] += 240; //on timer
		P1->OUT |= BIT0; //turn led on
	}
}

ii.) main.c with ISR for 20-second period clock system.

//******************************************************************************
//  MSP432P401 Demo - Timer0_A3, Toggle P1.0, CCR0 Cont Mode ISR, DCO SMCLK
//
//  Description: Toggle P1.0 using software and TA_0 ISR. Timer0_A is
//  configured for continuous mode, thus the timer overflows when TAR counts
//  to CCR0. In this example, CCR0 is loaded with 50000.
//  ACLK = n/a, MCLK = SMCLK = TACLK = default DCO = ~3MHz
//
//           MSP432P401x
//         ---------------
//     /|\|               |
//      | |               |
//      --|RST            |
//        |               |
//        |           P1.0|-->LED
//
//   William Goh
//   Texas Instruments Inc.
//   June 2016 (updated) | November 2013 (created)
//   Built with CCSv6.1, IAR, Keil, GCC
//******************************************************************************
#include "msp.h"
#define CYCLETIMER 60000 //Adjust this for ISR Timer

int numInterrupts;

int main(void) {
    WDT_A->CTL = WDT_A_CTL_PW |             // Stop WDT
            WDT_A_CTL_HOLD;

    // Configure LED
    P2->SEL1 &= ~BIT1;
    P2->SEL0 &= ~BIT1;
	P2->DIR |= BIT1;
	
	CS->KEY = CS_KEY_VAL; // unlock CS registers
    CS->CTL0 = 0; // clear register CTL0
	CS->CTL0=CS_CTL0_DCORSEL_0; //1.5 MHz
	CS->CTL1 = CS_CTL1_SELA_2 | CS_CTL1_SELS_3 | CS_CTL1_SELM_3; // select clock sources
    CS->KEY = 0; // lock the CS registers
	
	
	P2->OUT |= BIT1;
	
    TIMER_A0->CCTL[0] = TIMER_A_CCTLN_CCIE; // TACCR0 interrupt enabled
	
    TIMER_A0->CCR[0] = CYCLETIMER; //Set CCR0 for on cycle
	
    TIMER_A0->CTL = TIMER_A_CTL_SSEL__SMCLK | // SMCLK, continuous mode
            TIMER_A_CTL_MC__CONTINUOUS;

    SCB->SCR |= SCB_SCR_SLEEPONEXIT_Msk;    // Enable sleep on exit from ISR

    // Enable global interrupt
    __enable_irq();
    numInterrupts = 0;
	
    NVIC->ISER[0] = 1 << ((TA0_0_IRQn) & 31);

    while (1)
    {
        __sleep();

        __no_operation();                   // For debugger
    }
}

// Timer A0 interrupt service routine

void TA0_0_IRQHandler(void) {
    TIMER_A0->CCTL[0] &= ~TIMER_A_CCTLN_CCIFG;
	
	numInterrupts += 1;
	TIMER_A0->CCR[0] += CYCLETIMER; //increment timer
	
	if (numInterrupts == 250) {
		numInterrupts = 0;
		P2->OUT ^= BIT1; //turn led on/off
	}
}

iii.) main.c with for 2-bit counter generation

//******************************************************************************

//  MSP432P401 Demo - Timer0_A3, Toggle P1.0, CCR0 Cont Mode ISR, DCO SMCLK

//

//  Description: Toggle P1.0 using software and TA_0 ISR. Timer0_A is

//  configured for continuous mode, thus the timer overflows when TAR counts

//  to CCR0. In this example, CCR0 is loaded with 50000.

//  ACLK = n/a, MCLK = SMCLK = TACLK = default DCO = ~3MHz

//

//           MSP432P401x

//         ---------------

//     /|\|               |

//      | |               |

//      --|RST            |

//        |               |

//        |           P1.0|-->LED

//

//   William Goh

//   Texas Instruments Inc.

//   June 2016 (updated) | November 2013 (created)

//   Built with CCSv6.1, IAR, Keil, GCC

//******************************************************************************

#include "msp.h"

#define CYCLETIMER0 750 //Adjust this for lower bit

#define CYCLETIMER1 1500 //Adjust this for upper bit

int main(void) {

WDT_A->CTL = WDT_A_CTL_PW |             // Stop WDT

WDT_A_CTL_HOLD;

// Configure P5.0 P5.1

P5 -> DIR |= BIT0 | BIT1;

P5 -> OUT = 0;

CS->KEY = CS_KEY_VAL; // unlock CS registers

CS->CTL0 = 0; // clear register CTL0

CS->CTL0=CS_CTL0_DCORSEL_0; //1.5 MHz

CS->CTL1 = CS_CTL1_SELA_2 | CS_CTL1_SELS_3 | CS_CTL1_SELM_3; // select clock sources

CS->KEY = 0; // lock the CS registers

TIMER_A0->CCTL[0] = TIMER_A_CCTLN_CCIE; // TACCR0 interrupt enabled

TIMER_A0->CCTL[1] = TIMER_A_CCTLN_CCIE; // TACCR1 interrupt enabled

TIMER_A0->CCR[0] = CYCLETIMER0; //Set CCR0 for on cycle

TIMER_A0->CCR[1] = CYCLETIMER1; //SET CCR1 for cycle

TIMER_A0->CTL = TIMER_A_CTL_SSEL__SMCLK | // SMCLK, continuous mode

TIMER_A_CTL_MC__CONTINUOUS;

SCB->SCR |= SCB_SCR_SLEEPONEXIT_Msk;    // Enable sleep on exit from ISR

// Enable global interrupt

__enable_irq();

NVIC->ISER[0] = 1 << ((TA0_0_IRQn) & 31);

NVIC->ISER[0] = 1 << ((TA0_N_IRQn) & 31);

while (1)

{

__sleep();

__no_operation();                   // For debugger

}

}

// Timer A0 interrupt service routine

void TA0_0_IRQHandler(void) {

TIMER_A0->CCTL[0] &= ~TIMER_A_CCTLN_CCIFG;

TIMER_A0->CCR[0] += CYCLETIMER0; //increment timer

if (P5->OUT & BIT0)

P5 -> OUT &= ~BIT0;

else

P5-> OUT |= BIT0;

}

void TA0_N_IRQHandler(void) {

if (TIMER_A0->CCTL[1] & TIMER_A_CCTLN_CCIFG)

{

TIMER_A0->CCTL[1] &= ~TIMER_A_CCTLN_CCIFG;

TIMER_A0->CCR[1] += CYCLETIMER1; //increment timer

if (P5->OUT & BIT1)

P5 -> OUT &= ~BIT1;

else

P5-> OUT |= BIT1;

}

}